EXP – 10

Tanisha Gargade – 304D005

/* Includes ------------------------------------------------------------------*/

#include "main.h"

#include "lcd.h"

#include<stdio.h>

/* Private variables ---------------------------------------------------------*/

TIM_HandleTypeDef htim6;

/* Private function prototypes -----------------------------------------------*/

void SystemClock_Config(void);

static void MX_GPIO_Init(void);

static void MX_TIM6_Init(void);

void delay (uint16_t time)

{

/* change your code here for the delay in microseconds */

__HAL_TIM_SET_COUNTER(&htim6, 0);

while ((__HAL_TIM_GET_COUNTER(&htim6))<time);

}

/**

* @brief  The application entry point.

* @retval int

*/

int main(void)

{

unsigned int echo_time;

float distance;

unsigned char buf[16];

/* Reset of all peripherals, Initializes the Flash interface and the Systick. */

HAL_Init();

/* Configure the system clock */

SystemClock_Config();

/* Initialize all configured peripherals */

MX_GPIO_Init();

MX_TIM6_Init();

HAL_TIM_Base_Start(&htim6);

lcd_init();

lcd_clear();

/* Infinite loop */

/* USER CODE BEGIN WHILE */

while (1)

{

lcd_displayString(1,1,"HC-SR04");

HAL_Delay(100);

while((HAL_GPIO_ReadPin (ECHO_GPIO_Port, ECHO_Pin)==1));

//give rig pulse

HAL_GPIO_WritePin (TRIG_GPIO_Port, TRIG_Pin, 1);   // pull the pin high

delay(10);

HAL_GPIO_WritePin (TRIG_GPIO_Port, TRIG_Pin, 0);   // pull the pin low

//check echo signal going high

while((HAL_GPIO_ReadPin (ECHO_GPIO_Port, ECHO_Pin)==0));

//start counting ECHO high time

__HAL_TIM_SET_COUNTER(&htim6, 0);

while((HAL_GPIO_ReadPin (ECHO_GPIO_Port, ECHO_Pin)==1));

echo_time = __HAL_TIM_GET_COUNTER(&htim6);

if(echo_time >= 38000)

{

lcd_displayString(2,0,"no signal");

}

else

{

lcd_clear();

distance = (float)(echo_time*0.0343)/2;

sprintf(buf,"%3.2f cm",distance);

lcd_displayString(2,0,buf);

}   }

/* USER CODE END 3 */

}

/**

* @brief System Clock Configuration

* @retval None

*/

void SystemClock_Config(void)

{

RCC_OscInitTypeDef RCC_OscInitStruct = {0};

RCC_ClkInitTypeDef RCC_ClkInitStruct = {0};

/** Configure the main internal regulator output voltage

*/

__HAL_RCC_PWR_CLK_ENABLE();

__HAL_PWR_VOLTAGESCALING_CONFIG(PWR_REGULATOR_VOLTAGE_SCALE1);

/** Initializes the RCC Oscillators according to the specified parameters

* in the RCC_OscInitTypeDef structure.

*/

RCC_OscInitStruct.OscillatorType = RCC_OSCILLATORTYPE_HSI;

RCC_OscInitStruct.HSIState = RCC_HSI_ON;

RCC_OscInitStruct.HSICalibrationValue = RCC_HSICALIBRATION_DEFAULT;

RCC_OscInitStruct.PLL.PLLState = RCC_PLL_ON;

RCC_OscInitStruct.PLL.PLLSource = RCC_PLLSOURCE_HSI;

RCC_OscInitStruct.PLL.PLLM = 8;

RCC_OscInitStruct.PLL.PLLN = 168;

RCC_OscInitStruct.PLL.PLLP = RCC_PLLP_DIV2;

RCC_OscInitStruct.PLL.PLLQ = 4;

if (HAL_RCC_OscConfig(&RCC_OscInitStruct) != HAL_OK)

{

Error_Handler();

}

/** Initializes the CPU, AHB and APB buses clocks

*/

RCC_ClkInitStruct.ClockType = RCC_CLOCKTYPE_HCLK|RCC_CLOCKTYPE_SYSCLK

|RCC_CLOCKTYPE_PCLK1|RCC_CLOCKTYPE_PCLK2;

RCC_ClkInitStruct.SYSCLKSource = RCC_SYSCLKSOURCE_PLLCLK;

RCC_ClkInitStruct.AHBCLKDivider = RCC_SYSCLK_DIV1;

RCC_ClkInitStruct.APB1CLKDivider = RCC_HCLK_DIV4;

RCC_ClkInitStruct.APB2CLKDivider = RCC_HCLK_DIV2;

if (HAL_RCC_ClockConfig(&RCC_ClkInitStruct, FLASH_LATENCY_5) != HAL_OK)

{

Error_Handler();

}    }

/**

* @brief TIM6 Initialization Function

* @param None

* @retval None

*/

static void MX_TIM6_Init(void)

{

/* USER CODE BEGIN TIM6_Init 0 */

/* USER CODE END TIM6_Init 0 */

TIM_MasterConfigTypeDef sMasterConfig = {0};

/* USER CODE BEGIN TIM6_Init 1 */

/* USER CODE END TIM6_Init 1 */

htim6.Instance = TIM6;

htim6.Init.Prescaler = 84;

htim6.Init.CounterMode = TIM_COUNTERMODE_UP;

htim6.Init.Period = 65535;

htim6.Init.AutoReloadPreload = TIM_AUTORELOAD_PRELOAD_ENABLE;

if (HAL_TIM_Base_Init(&htim6) != HAL_OK)

{   Error_Handler();   }

sMasterConfig.MasterOutputTrigger = TIM_TRGO_RESET;

sMasterConfig.MasterSlaveMode = TIM_MASTERSLAVEMODE_DISABLE;

if (HAL_TIMEx_MasterConfigSynchronization(&htim6, &sMasterConfig) != HAL_OK)

{   Error_Handler();  }

/* USER CODE BEGIN TIM6_Init 2 */

/* USER CODE END TIM6_Init 2 */    }

/**

* @brief GPIO Initialization Function

* @param None

* @retval None

*/

static void MX_GPIO_Init(void)

{         GPIO_InitTypeDef GPIO_InitStruct = {0};

/* GPIO Ports Clock Enable */

__HAL_RCC_GPIOD_CLK_ENABLE();

/*Configure GPIO pin Output Level */

HAL_GPIO_WritePin(TRIG_GPIO_Port, TRIG_Pin, GPIO_PIN_RESET);

/*Configure GPIO pin : TRIG_Pin */

GPIO_InitStruct.Pin = TRIG_Pin;

GPIO_InitStruct.Mode = GPIO_MODE_OUTPUT_PP;

GPIO_InitStruct.Pull = GPIO_NOPULL;

GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_LOW;

HAL_GPIO_Init(TRIG_GPIO_Port, &GPIO_InitStruct);

/*Configure GPIO pin : ECHO_Pin */

GPIO_InitStruct.Pin = ECHO_Pin;

GPIO_InitStruct.Mode = GPIO_MODE_INPUT;

GPIO_InitStruct.Pull = GPIO_NOPULL;

HAL_GPIO_Init(ECHO_GPIO_Port, &GPIO_InitStruct);

}

/* USER CODE BEGIN 4 */

/* USER CODE END 4 */

/**

* @brief  This function is executed in case of error occurrence.

* @retval None

*/

void Error_Handler(void)

{

/* USER CODE BEGIN Error_Handler_Debug */

/* User can add his own implementation to report the HAL error return state */

__disable_irq();

while (1)

{   }

/* USER CODE END Error_Handler_Debug */    }

#ifdef  USE_FULL_ASSERT

/**

* @brief  Reports the name of the source file and the source line number

*         where the assert_param error has occurred.

* @param  file: pointer to the source file name

* @param  line: assert_param error line source number

* @retval None

*/

void assert_failed(uint8_t *file, uint32_t line)

{

/* USER CODE BEGIN 6 */

/* User can add his own implementation to report the file name and line number,

ex: printf("Wrong parameters value: file %s on line %d\r\n", file, line) */

/* USER CODE END 6 */

}

#endif /* USE_FULL_ASSERT */