EXP – 02 (a) – LED loop

Tanisha Gargade – 304D005

Code:

#include<lpc214x.h>

#define LED0  12

void delay(unsigned int time)

{

unsigned int i,j;

for(i = 0; i < time ;i++ )

{

for(j = 0; j < 5000 ; j++);

}

}

int main(void)

{

int i,j;

PINSEL0 = 0x00000000;  //set the pin function for pins as GPIO

IODIR0 = 0x000FF000;  // set the direction of pins as  output

while(1)

{

for(i = 0; i < 4 ; i++)

{

IOSET0 = 0x000FF000;			// set the port pins to 1

delay(150);

IOCLR0 = 0x000FF000;			//clear the port pins to 0

delay(150);

}

//LED Rolling

for(i = 0 ; i < 4 ; i++)

{

for(j = 0 ; j < 8; j++)

{

IOSET0 = 1 << (LED0 + j);	 // set the port pins to 1

delay(50);

}

for(j = 7 ; j >= 0; j--)

{

IOCLR0 = 1 << (LED0 + j); //clear the port pins to 0

delay(50);

}

}

}

return 0;

}