EXP – 3(a) – Main ADC

Tanisha Gargade – 304D005

Code:

#include <LPC214x.h>

#define ADC0 1 << 24

#define VREF 3.3

int main(void)

{

int i,j;

int unsigned long result;

unsigned char key;

PINSEL1 = (1<<24);// INITIALIZE ADC0 CH0 AD0.1 (p0.28)

UartInit();

AD0CR = 0X01210302;

// Channel 0, Clock 4Mhz, Burst Mode,10 bit,PDN = 1,

while(1)

{

while(!(AD0DR1 & 0X80000000)); // wait for conversion

result = ((AD0DR1>>6) & 0x3ff);

printf("\nVoltage on AD0.1 = %dmV\n",(int)( result*VREF ));

for(j=9;j>=0;j--)

{

key=(result>>j)&0x001; // fetching MSB to LSB bits at a time

UART_PutChar(key+0x30);// conversion to ASCII

}

for(i=0;i<10000;i++)

for(j=0;j<5000;j++);

}

return 0;

}