EXP – 09

Tanisha Gargade – 304D005

/* Includes ------------------------------------------------------------------*/

#include "main.h"

#include "lcd.h"

#include <stdio.h>

/* Private variables ---------------------------------------------------------*/

I2C_HandleTypeDef hi2c1;

/* USER CODE BEGIN PV */

/* USER CODE END PV */

/* Private function prototypes -----------------------------------------------*/

void SystemClock_Config(void);

static void MX_GPIO_Init(void);

static void MX_I2C1_Init(void);

#define MPU6050_ADDR 0xD0

#define SMPLRT_DIV_REG 0x19

#define GYRO_CONFIG_REG 0x1B

#define ACCEL_CONFIG_REG 0x1C

#define ACCEL_XOUT_H_REG 0x3B

#define TEMP_OUT_H_REG 0x41

#define GYRO_XOUT_H_REG 0x43

#define PWR_MGMT_1_REG 0x6B

#define WHO_AM_I_REG 0x75

int16_t Accel_X_RAW = 0;

int16_t Accel_Y_RAW = 0;

int16_t Accel_Z_RAW = 0;

int16_t Gyro_X_RAW = 0;

int16_t Gyro_Y_RAW = 0;

int16_t Gyro_Z_RAW = 0;

float Ax, Ay, Az, Gx, Gy, Gz;

void MPU6050_Init (void)

{

uint8_t check;

uint8_t Data;

HAL_I2C_Mem_Read (&hi2c1, MPU6050_ADDR,WHO_AM_I_REG,1, &check, 1, 1000);

if (check == 104)  // 0x68 will be returned by the sensor if everything goes well

{

// power management register 0X6B we should write all 0's to wake the sensor up

Data = 0;

HAL_I2C_Mem_Write(&hi2c1, MPU6050_ADDR, PWR_MGMT_1_REG, 1,&Data, 1, 1000);

// Set DATA RATE of 1KHz by writing SMPLRT_DIV register

Data = 0x07;

HAL_I2C_Mem_Write(&hi2c1, MPU6050_ADDR, SMPLRT_DIV_REG, 1, &Data, 1, 1000);

// Set accelerometer configuration in ACCEL_CONFIG Register

// XA_ST=0,YA_ST=0,ZA_ST=0, FS_SEL=0 -> Â± 2g

Data = 0x00;

HAL_I2C_Mem_Write(&hi2c1, MPU6050_ADDR, ACCEL_CONFIG_REG, 1, &Data, 1, 1000);

// Set Gyroscopic configuration in GYRO_CONFIG Register

// XG_ST=0,YG_ST=0,ZG_ST=0, FS_SEL=0 -> Â± 250 Â°/s

Data = 0x00;

HAL_I2C_Mem_Write(&hi2c1, MPU6050_ADDR, GYRO_CONFIG_REG, 1, &Data, 1, 1000);

}   }

void MPU6050_Read_Accel (void)

{

uint8_t Rec_Data[6];

// Read 6 BYTES of data starting from ACCEL_XOUT_H register

HAL_I2C_Mem_Read (&hi2c1, MPU6050_ADDR, ACCEL_XOUT_H_REG, 1, Rec_Data, 6, 1000);

Accel_X_RAW = (int16_t)(Rec_Data[0] << 8 | Rec_Data [1]);

Accel_Y_RAW = (int16_t)(Rec_Data[2] << 8 | Rec_Data [3]);

Accel_Z_RAW = (int16_t)(Rec_Data[4] << 8 | Rec_Data [5]);

/*** convert the RAW values into acceleration in 'g'

we have to divide according to the Full scale value set in FS_SEL

I have configured FS_SEL = 0. So I am dividing by 16384.0

for more details check ACCEL_CONFIG Register              ****/

Ax = Accel_X_RAW/16384.0;

Ay = Accel_Y_RAW/16384.0;

Az = Accel_Z_RAW/16384.0;

}

void MPU6050_Read_Gyro (void)

{	uint8_t Rec_Data[6];

// Read 6 BYTES of data starting from GYRO_XOUT_H register

HAL_I2C_Mem_Read (&hi2c1, MPU6050_ADDR, GYRO_XOUT_H_REG, 1, Rec_Data, 6, 1000);

Gyro_X_RAW = (int16_t)(Rec_Data[0] << 8 | Rec_Data [1]);

Gyro_Y_RAW = (int16_t)(Rec_Data[2] << 8 | Rec_Data [3]);

Gyro_Z_RAW = (int16_t)(Rec_Data[4] << 8 | Rec_Data [5]);

/*** convert the RAW values into dps (Â°/s)

we have to divide according to the Full scale value set in FS_SEL

I have configured FS_SEL = 0. So I am dividing by 131.0

for more details check GYRO_CONFIG Register              ****/

Gx = Gyro_X_RAW/131.0;

Gy = Gyro_Y_RAW/131.0;

Gz = Gyro_Z_RAW/131.0;

}

/**

* @brief  The application entry point.

* @retval int

*/

int main(void)

{             char buf[4];

/* Reset of all peripherals, Initializes the Flash interface and the Systick. */

HAL_Init();

/* Configure the system clock */

SystemClock_Config();

/* Initialize all configured peripherals */

MX_GPIO_Init();

MX_I2C1_Init();

lcd_init();

lcd_clear();

MPU6050_Init();

lcd_displayString(1,1,"MPU6050");

HAL_Delay(1000);

/* USER CODE BEGIN WHILE */

while (1)

{             MPU6050_Read_Accel();

MPU6050_Read_Gyro();

// print the Acceleration and Gyro values on the LCD 20x4

lcd_displayString(1,0,"Ax=");

sprintf (buf, "%.2f", Ax);

lcd_displayString (1,3,buf);

lcd_displayString (1,10,"g");/*

// lcd_displayString(2,0,"Gx=");

// sprintf (buf, "%.2f", Gx);

// lcd_displayString (2,3,buf);

HAL_Delay (1000);  // wait for a while

// lcd_clear();

lcd_displayString(1,0,"Ay=");

sprintf (buf, "%.2f", Ay);

lcd_displayString (1,3,buf);

lcd_displayString (1,10,"g");

//lcd_displayString(2,0,"Gy=");

//sprintf (buf, "%.2f", Gy);

//lcd_displayString (2,3,buf);

HAL_Delay (1000);  // wait for a while

//lcd_clear();

//lcd_displayString(1,0,"Az=");

//sprintf (buf, "%.2f", Az);

//lcd_displayString (1,3,buf);

//lcd_displayString (1,10,"g");

//lcd_displayString(2,0,"Gz=");

//sprintf (buf, "%.2f", Gz);

//lcd_displayString (2,3,buf);

//HAL_Delay (1000);  // wait for a while

//lcd_clear();*/    }

/* USER CODE END 3 */  }

/**

* @brief System Clock Configuration

* @retval None

*/

void SystemClock_Config(void)

{   RCC_OscInitTypeDef RCC_OscInitStruct = {0};

RCC_ClkInitTypeDef RCC_ClkInitStruct = {0};

/** Configure the main internal regulator output voltage

*/

__HAL_RCC_PWR_CLK_ENABLE();

__HAL_PWR_VOLTAGESCALING_CONFIG(PWR_REGULATOR_VOLTAGE_SCALE1);

/** Initializes the RCC Oscillators according to the specified parameters

* in the RCC_OscInitTypeDef structure.

*/

RCC_OscInitStruct.OscillatorType = RCC_OSCILLATORTYPE_HSI;

RCC_OscInitStruct.HSIState = RCC_HSI_ON;

RCC_OscInitStruct.HSICalibrationValue = RCC_HSICALIBRATION_DEFAULT;

RCC_OscInitStruct.PLL.PLLState = RCC_PLL_ON;

RCC_OscInitStruct.PLL.PLLSource = RCC_PLLSOURCE_HSI;

RCC_OscInitStruct.PLL.PLLM = 16;

RCC_OscInitStruct.PLL.PLLN = 336;

RCC_OscInitStruct.PLL.PLLP = RCC_PLLP_DIV2;

RCC_OscInitStruct.PLL.PLLQ = 4;

if (HAL_RCC_OscConfig(&RCC_OscInitStruct) != HAL_OK)

{  Error_Handler();   }

/** Initializes the CPU, AHB and APB buses clocks

*/

RCC_ClkInitStruct.ClockType = RCC_CLOCKTYPE_HCLK|RCC_CLOCKTYPE_SYSCLK

|RCC_CLOCKTYPE_PCLK1|RCC_CLOCKTYPE_PCLK2;

RCC_ClkInitStruct.SYSCLKSource = RCC_SYSCLKSOURCE_PLLCLK;

RCC_ClkInitStruct.AHBCLKDivider = RCC_SYSCLK_DIV1;

RCC_ClkInitStruct.APB1CLKDivider = RCC_HCLK_DIV4;

RCC_ClkInitStruct.APB2CLKDivider = RCC_HCLK_DIV2;

if (HAL_RCC_ClockConfig(&RCC_ClkInitStruct, FLASH_LATENCY_5) != HAL_OK)

{  Error_Handler();    }    }

/**

* @brief I2C1 Initialization Function

* @param None

* @retval None

*/

static void MX_I2C1_Init(void)

{    /* USER CODE BEGIN I2C1_Init 0 */

/* USER CODE END I2C1_Init 0 */

/* USER CODE BEGIN I2C1_Init 1 */

/* USER CODE END I2C1_Init 1 */

hi2c1.Instance = I2C1;

hi2c1.Init.ClockSpeed = 400000;

hi2c1.Init.DutyCycle = I2C_DUTYCYCLE_2;

hi2c1.Init.OwnAddress1 = 0;

hi2c1.Init.AddressingMode = I2C_ADDRESSINGMODE_7BIT;

hi2c1.Init.DualAddressMode = I2C_DUALADDRESS_DISABLE;

hi2c1.Init.OwnAddress2 = 0;

hi2c1.Init.GeneralCallMode = I2C_GENERALCALL_DISABLE;

hi2c1.Init.NoStretchMode = I2C_NOSTRETCH_DISABLE;

if (HAL_I2C_Init(&hi2c1) != HAL_OK)

{

Error_Handler();

}

/* USER CODE BEGIN I2C1_Init 2 */

/* USER CODE END I2C1_Init 2 */

}

/**

* @brief GPIO Initialization Function

* @param None

* @retval None

*/

static void MX_GPIO_Init(void)

{

/* GPIO Ports Clock Enable */

__HAL_RCC_GPIOB_CLK_ENABLE();

}

/* USER CODE BEGIN 4 */

/* USER CODE END 4 */

/**

* @brief  This function is executed in case of error occurrence.

* @retval None

*/

void Error_Handler(void)

{

/* USER CODE BEGIN Error_Handler_Debug */

/* User can add his own implementation to report the HAL error return state */

__disable_irq();

while (1)

{  }

/* USER CODE END Error_Handler_Debug */

}

#ifdef  USE_FULL_ASSERT

/**

* @brief  Reports the name of the source file and the source line number

*         where the assert_param error has occurred.

* @param  file: pointer to the source file name

* @param  line: assert_param error line source number

* @retval None

*/

void assert_failed(uint8_t *file, uint32_t line)

{

/* USER CODE BEGIN 6 */

/* User can add his own implementation to report the file name and line number,

ex: printf("Wrong parameters value: file %s on line %d\r\n", file, line) */

/* USER CODE END 6 */

}

#endif /* USE_FULL_ASSERT */